How to Turn Off USB Power Saving Function on the D25 NC and D25 NC FM Printer Option

- Turn off the power and remove the Power Cord.

- Turn the printer over and using a Phillips screw driver remove the Dip Switch Access cover.

- Set DSW1, SW1-8 to ON.

- Replace the Dip Switch Access cover.

- The new switch setting becomes effective upon power up.

- This keeps the printer from going in to power save mode shutting down the USB communications port.